V2V Network Simulator - Project Brief

EXECUTIVE SUMMARY

Project Goal: Build a visual simulator demonstrating how autonomous vehicles communicate safety-critical information in real-time using vehicle-to-vehicle (V2V) networking protocols.

Core Question: How do mobile ad-hoc networks (MANETs) handle ultra-low latency requirements when network topology changes every second at highway speeds?

Success Metric: Live demo showing 10+ vehicles successfully preventing collisions through V2V communication with <10ms message latency and zero critical message loss.

FUNCTIONAL REQUIREMENTS

Must Demonstrate (90 points)

- Vehicle Simulation (50 points):

  - 10+ vehicles moving in either highway (60+ mph) or city (25-35 mph) scenario with realistic acceleration/deceleration

  - Demonstrate at least one collision avoidance scenario

- Message Protocol (20 points):

  - Basic Safety Messages (BSM): Each vehicle broadcasts its position, speed, and heading every 100ms (10 times per second). This constant "heartbeat" lets nearby vehicles track each other's movements

  - Emergency Collision Warnings (CWM): When a vehicle detects an imminent collision (Time-To-Collision < 3 seconds), it immediately sends a high-priority emergency message that preempts regular BSM traffic. These messages trigger automatic warning alerts in nearby vehicles

  - Message Priority: Emergency messages must be transmitted within 5ms and never dropped, while BSM messages can tolerate occasional loss

- Network Behavior: 300m communication range, <10ms average latency for safety messages, configurable packet loss (0-10%) to simulate interference

- Live Visualization (20 points) -

  - Live-updating table showing all vehicle states (ID, position, speed, heading)

  - Message log displaying recent BSM and CWM transmissions with timestamps

  - Network connectivity matrix showing which vehicles can communicate

  - Statistics panel: active vehicles, messages/second, latency, collisions prevented

  - Text-based alerts when collision warnings are triggered

ARCHITECTURE & TECHNOLOGY

Recommended Stack

┌─────────────────────────────┐

│    Visualization Layer      │  ← Pygame/Canvas OR Terminal/Web UI

├─────────────────────────────┤

│    Simulation Engine        │  ← SimPy (Python) or Custom (JS)

├──────────────┬──────────────┤

│   Vehicle    │   Network    │

│   Physics    │   Protocol   │

└──────────────┴──────────────┘

Python Option:

- simpy - Discrete event simulation

- pygame (graphical) OR rich/curses (tabular) - Visualization

- numpy - Physics calculations

JavaScript Option:

- Canvas API (graphical) OR HTML tables (tabular) - Visualization

- Custom event loop - Simulation

Key Components

- Vehicle Class: Position, velocity, acceleration, message queue

- Protocol Handler: Message prioritization, retransmission, authentication stub

- Network Simulator: Range checking, latency modeling, packet loss

- Visualization Engine: Real-time rendering at 30 FPS (graphical) or 10 updates/sec (tabular)

ASSUMPTIONS & CONSTRAINTS

Simulation Assumptions

- Vehicles follow predefined paths (straight lines or simple curves - no complex steering)

- Network is simulated via message passing (no actual wireless hardware)

- Perfect positioning (no GPS error modeling required)

- Simplified collision detection using bounding boxes

- Message authentication is mocked with simple IDs (no cryptography implementation needed)

Performance Targets

- Support 10-15 vehicles simultaneously

- Maintain 30 FPS (graphical) or 10 updates/sec (tabular) visualization

- Run on standard laptop (<300MB RAM, <50% CPU)

- Simulation speed: 1x real-time (optional: 2-5x speedup)

Scope Limitations

- 2D visualization only (3D is bonus)

- Single road scenario (highway or city intersection)

- No vehicle-to-infrastructure communication

- No weather/terrain effects on signal propagation